VLSI BUG INTELLIGENCE PLATFORM

VBIP — Architecture & Implementation Specification

Sections 3–4: Checker Network · Log Parser Engine

Classification: Internal — Engineering Confidential

Version 1.0  |  AMS · DMS · Mixed-Signal · Digital Domains

# SECTION 3 — EXHAUSTIVE CHECKER NETWORK SPECIFICATION

This section defines the complete checker library for AMS, Digital/DMS, and Mixed-Signal Interface domains. Each checker entry specifies the monitored signal type, threshold/condition, severity mapping, and the exact auto-prefill fields emitted to the VBIP bug template when the checker fires.

Architectural Principle Checkers are the primary automation driver for bug filing. A checker that fires but cannot populate a meaningful prefill payload wastes the DV engineer's time. Every checker in this library is designed to emit a complete, ready-to-submit auto_prefill_fields payload. The DV engineer's role is verification and context enrichment — not data re-entry.

## 3.1  Checker Coverage Map

| Domain Category | # Checkers | S1 Count | S2 Count | Key Failure Modes Covered |
| --- | --- | --- | --- | --- |
| AMS — Supply Rail | 4 | 2 | 2 | Undervoltage, overvoltage, ripple, PSRR |
| AMS — LDO/Regulator | 4 | 2 | 2 | Dropout, load/line transient, stability |
| AMS — PLL/Oscillator | 4 | 1 | 3 | Lock time, jitter, frequency, VCO clipping |
| AMS — Current Monitor | 3 | 0 | 2 | Quiescent, inrush, leakage |
| AMS — OTA/Opamp | 4 | 0 | 4 | Slew rate, settling, offset, CMRR |
| AMS — ESD/Protection | 2 | 2 | 0 | Clamp activation, latchup risk |
| AMS — Bias Networks | 2 | 0 | 2 | VREF drift, PTAT/CTAT deviation |
| AMS — Monte Carlo | 2 | 0 | 1 | 3σ yield violation, 6σ margin alert |
| Digital — Protocol (AHB/APB/AXI) | 6 | 1 | 5 | All handshake, ordering, strobe rules |
| Digital — CDC | 3 | 2 | 1 | Metastability, missing sync, gray code |
| Digital — Reset | 3 | 0 | 2 | Async deassert, fanout, partial reset |
| Digital — FSM | 3 | 1 | 2 | Illegal state, undefined transition, lockout |
| Digital — Timing (SDF) | 3 | 2 | 0 | Setup, hold violations, glitches |
| Digital — Power | 3 | 0 | 2 | Dynamic spike, multi-Vt, isolation cell |
| Digital — Functional | 3 | 1 | 2 | Counter overflow, FIFO, ECC/parity |
| Digital — SVA Coverage | 2 | 0 | 1 | Vacuity, never-triggered properties |
| Mixed-Signal — ADC | 4 | 1 | 3 | DNL, INL, missing code, conversion glitch |
| Mixed-Signal — DAC | 2 | 0 | 2 | Settling, glitch energy |
| Mixed-Signal — Comparator | 2 | 1 | 1 | Metastability window, hysteresis |
| Mixed-Signal — Level Shifter | 1 | 1 | 0 | Undefined logic level in transition band |
| Mixed-Signal — Handshake | 1 | 0 | 1 | Analog-digital ready/valid timing |
| TOTAL | 61 | 17 | 38 | Full AMS + Digital + Mixed-Signal coverage |

## 3.2  AMS Checker Definitions

| ANALOG / MIXED-SIGNAL (AMS) CHECKERS — CHK_SUP / LDO / PLL / IQ / OTA / ESD / BIAS / MC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Checker ID / Name | Monitored Signal(s) | Threshold / Condition | Sev | Domain | Auto-Prefill Fields |
| CHK_SUP_UV_001 Supply Undervoltage | VDD, VDDA, AVDD rails | V_rail < V_nom × (1 − tolerance%) Default tolerance: ±5% Hysteresis: 20mV Min window: 10ns | S1 | AMS | domain=AMS, bug_type=Functional, title=[{block}][Supply] Rail undervoltage detected, primary_signals=[{rail_name}], observed_behavior=Rail at {obs_v}V (spec_min={spec_v}V), suggested_root_cause=Margin_Violation |
| CHK_SUP_OV_002 Supply Overvoltage | VDD, VDDA rails | V_rail > V_nom × (1 + tolerance%) Default tolerance: ±5% Latch-out after 100ns sustained | S1 | AMS | severity=S1-Critical, bug_type=Power, title=[{block}][Supply] Rail overvoltage – potential oxide stress, failure_conditions.voltage={obs_v}V |
| CHK_SUP_RIPPLE_003 Ripple Excess | VDD AC component (HP-filtered) | Peak-to-peak ripple > ripple_spec Measure over 10 switching cycles FFT: harmonic > −40dBc from DC | S2 | AMS | bug_type=Noise_Margin, description.observed_behavior=Ripple {pp_mv}mVpp (spec: {spec_mv}mVpp), t_start={t0}, t_end={t1} |
| CHK_SUP_PSRR_004 PSRR Violation | VOUT vs VIN AC transfer function | PSRR_meas(f) < PSRR_spec(f) Checked at: 100Hz, 1kHz, 10kHz, 100kHz Margin threshold: 3dB | S2 | AMS | bug_type=Noise_Margin, checker_name=CHK_SUP_PSRR_004, design_phase=Post-layout, title=[{block}][PSRR] Rejection below spec at {freq}Hz |
| CHK_LDO_DROP_005 Dropout Violation | V_IN − V_OUT differential | V_dropout < V_dropout_min_spec At max load current Corner: worst-case tt/low-VDD/high-T | S1 | AMS | domain=AMS, bug_type=Functional, title=[LDO][Dropout] Insufficient headroom at max load, failure_conditions.voltage=VIN={vin}V VOUT={vout}V load={iload}mA |
| CHK_LDO_LOAD_TRANS_006 Load Transient | VOUT during load step | ΔV_out > undershoot_spec OR T_settle > settle_time_spec Load step: I_min→I_max, 1ns edge | S2 | AMS | bug_type=Functional, t_start={t_step}, t_peak={t_worst}, observed_behavior=Undershoot={undershoot}mV settle={settle}us |
| CHK_LDO_LINE_TRANS_007 Line Transient | VOUT during VIN step | ΔV_out > line_transient_spec VIN step: ±10% of nominal, 100ns ramp | S2 | AMS | bug_type=Functional, title=[LDO][LineTransient] Output deviation exceeds spec, primary_signals=[vout, vin] |
| CHK_LDO_STAB_008 Stability Proxy (Phase Margin) | VOUT AC response (open-loop equiv.) | Phase margin proxy < 45° Gain margin < 10dB Derived from closed-loop step response: damping ratio < 0.5 | S1 | AMS | bug_type=Functional, root_cause_category=Design_Intent_Misunderstanding, title=[LDO][Stability] Insufficient phase margin proxy – oscillation risk |
| CHK_PLL_LOCK_009 Lock Time Violation | PLL lock detect output, FOUT | T_lock > T_lock_spec Measure: PLL_EN assertion → lock_detect HIGH | S2 | AMS | bug_type=Timing, title=[PLL][LockTime] Lock time exceeds specification, t_peak={t_lock_actual} |
| CHK_PLL_JITTER_010 Jitter Excess | PLL output clock period variation | RMS jitter > jitter_rms_spec Peak-to-peak > jitter_pp_spec Measure over 10K cycles post-lock | S1 | AMS | severity=S1-Critical, bug_type=Noise_Margin, title=[PLL][Jitter] Output jitter exceeds spec, observed_behavior=RMS={jrms}ps pp={jpp}ps |
| CHK_PLL_FREQ_011 Frequency Out-of-Spec | FOUT measured post-lock | ABS(F_out − F_target) / F_target > 100ppm Or per-spec tolerance | S2 | AMS | bug_type=Functional, title=[PLL][Frequency] Output frequency deviation {ppm}ppm, observed_behavior=Fout={fout}MHz target={ftgt}MHz |
| CHK_PLL_VCO_CLIP_012 VCO Rail Clipping | VCTRL voltage | VCTRL > VDD − 0.1V OR VCTRL < 0.1V Indicates loop unlocked or gain mismatch | S2 | AMS | bug_type=Functional, title=[PLL][VCO] Control voltage clipping – loop cannot acquire lock |
| CHK_IQ_EXCESS_013 Quiescent Current Excess | IDD supply current in standby mode | I_quiescent > IQ_spec × 1.1 (10% margin) Measure after 1ms settle All functional blocks idle | S2 | AMS | bug_type=Power, title=[{block}][IQ] Quiescent current exceeds specification, observed_behavior=IQ={iq_ua}uA spec={iq_spec}uA |
| CHK_INRUSH_014 Inrush Current Excess | IDD during power-on ramp | Peak inrush > inrush_spec Measure from VDDOK assertion → 95% of VDD | S2 | AMS | bug_type=Power, title=[{block}][Inrush] Power-on inrush current exceeds limit, t_start={t_pon}, failure_conditions.current={i_peak}mA |
| CHK_LEAK_015 Leakage Current | Reverse diode / bias leakage paths | I_leak > I_leak_spec at T=125°C Measure: all inputs at VDD/GND, outputs open | S3 | AMS | bug_type=Power, design_phase=Post-layout, title=[{block}][Leakage] Leakage current above spec at high temperature |
| CHK_OTA_SLEW_016 Slew Rate Violation | VOUT rate of change | SR_meas < SR_spec × 0.9 (10% margin) Measure: 10%–90% output swing Both rising and falling edges | S2 | AMS | bug_type=Functional, title=[OTA][SlewRate] Output slew rate below spec, observed_behavior=SR={sr_mvus}V/us spec={sr_spec}V/us |
| CHK_OTA_SETTLE_017 Settling Time | VOUT error band | T_settle > settle_spec Error band: 0.1% of final value (or per spec) Step input: 50% of full swing | S2 | AMS | bug_type=Functional, t_start={t_step}, t_end={t_settled}, title=[OTA][Settling] Output settling time exceeds spec |
| CHK_OTA_OFFSET_018 Input Offset Voltage | VOS = VOUT/Aol at DC | ABS(VOS) > VOS_spec (3σ budget) Monte Carlo: sigma violation tracked separately | S2 | AMS | bug_type=Parametric, title=[OTA][Offset] Input offset voltage exceeds spec, observed_behavior=Vos={vos_mv}mV spec={vos_spec}mV |
| CHK_OTA_CMRR_019 CMRR/PSRR | Differential gain vs common-mode | CMRR_meas(f) < CMRR_spec(f) OR PSRR_meas(f) < PSRR_spec(f) At DC and 1kHz | S2 | AMS | bug_type=Noise_Margin, title=[OTA][CMRR] Common-mode rejection below specification |
| CHK_ESD_CLAMP_020 ESD Clamp Activation | ESD clamp node current / voltage | Clamp current > 0 during normal operation (False trigger in functional mode) OR clamp fails to activate during ESD event | S1 | AMS | severity=S1-Critical, bug_type=Functional, title=[ESD][Clamp] Protection clamp triggered during functional operation – latchup risk |
| CHK_ESD_LATCHUP_021 Latchup Risk Signal | PNPN current path (substrate current) | I_substrate > latchup_trigger_threshold Occurs during fast supply transient Temperature: >85°C | S1 | AMS | severity=S1-Critical, bug_type=Functional, root_cause_category=Design_Intent_Misunderstanding, title=[ESD][Latchup] Substrate current indicates latchup risk |
| CHK_BIAS_VREF_022 VREF Drift | Bandgap / reference voltage output | ABS(VREF − VREF_nom) > VREF_spec_ppm × VREF_nom Over temperature range: −40°C to 125°C | S2 | AMS | bug_type=Parametric, title=[Bias][VREF] Reference voltage drift exceeds spec, failure_conditions.temperature={temp}C |
| CHK_BIAS_PTAT_023 PTAT/CTAT Deviation | PTAT / CTAT current branches | TC coefficient deviates > ±5% from nominal Extracted from two-temperature simulation | S2 | AMS | bug_type=Parametric, title=[Bias][PTAT] Temperature coefficient deviation – bandgap compensation error |
| CHK_MC_3SIGMA_024 Monte Carlo 3σ Violation | Any specified performance metric | Metric fails spec in > 0.27% of runs (3σ boundary) N_runs ≥ 1000 Track: yield estimate, sigma of worst metric | S2 | AMS | bug_type=Parametric, title=[{block}][MonteCarlo] 3σ yield violation on {metric_name}, observed_behavior=Yield={yield_pct}% (spec: 99.73%) |
| CHK_MC_6SIGMA_025 6σ Band Alert | Any specified performance metric | Metric fails spec beyond 3σ but within 6σ Flags potential manufacturing risk N_runs ≥ 3000 for confidence | S3 | AMS | bug_type=Parametric, severity=S3-Medium, title=[{block}][MonteCarlo] 6σ margin alert on {metric_name} |

## 3.3  Digital / DMS Checker Definitions

| DIGITAL / DMS CHECKERS — CHK_AHB / APB / AXI / CDC / RST / FSM / TIM / PWR / FUNC / SVA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Checker ID / Name | Monitored Signal(s) | Threshold / Condition | Sev | Domain | Auto-Prefill Fields |
| CHK_AHB_HTRANS_101 AHB HTRANS Illegal | HTRANS, HREADY, HSEL, HADDR | HTRANS=BUSY when master owns bus OR HTRANS=SEQ without prior NONSEQ OR HADDR changes during BUSY | S2 | Digital | domain=Digital, bug_type=Protocol, title=[AHB][Protocol] Illegal HTRANS sequence detected, primary_signals=[HTRANS, HREADY, HADDR] |
| CHK_AHB_SPLIT_102 AHB SPLIT/RETRY Abuse | HRESP, HREADY | HRESP=SPLIT without HMASTLOCK support OR >16 consecutive RETRY responses (livelock condition) | S2 | Digital | bug_type=Protocol, title=[AHB][SPLIT] Retry/SPLIT protocol violation – potential livelock |
| CHK_APB_SETUP_103 APB Setup Violation | PSEL, PENABLE, PADDR, PWRITE | PENABLE asserted without prior PSEL OR PADDR/PWRITE changes during ENABLE phase OR PSEL deasserted before PENABLE | S2 | Digital | bug_type=Protocol, title=[APB][Setup] APB enable phase protocol violation |
| CHK_AXI_HANDSHAKE_104 AXI Valid/Ready Deadlock | AWVALID, AWREADY, WVALID, WREADY, ARVALID, ARREADY | VALID asserted without READY response within N_timeout cycles Default N_timeout=256 OR READY deasserted while VALID held (AXI3 violation) | S1 | Digital | severity=S1-Critical, bug_type=Protocol, title=[AXI][Handshake] Valid/Ready deadlock detected, t_start={t_valid_assert} |
| CHK_AXI_ID_ORDER_105 AXI ID Ordering Violation | RID, BID, ARID, AWID transaction IDs | Responses out-of-order for same ID OR BID does not match AWID OR RID does not match ARID | S2 | Digital | bug_type=Protocol, title=[AXI][Ordering] Response ID ordering violation, observed_behavior=Expected_ID={exp_id} Got_ID={got_id} |
| CHK_AXI_STROBE_106 AXI Write Strobe Invalid | WSTRB signal vs WDATA | WSTRB pattern invalid for transfer size OR WSTRB=0 during WVALID (wasteful write) OR narrow transfer strobe misaligned to address | S3 | Digital | bug_type=Protocol, title=[AXI][WSTRB] Write strobe invalid for transfer width |
| CHK_CDC_META_201 Metastability Window Violation | Flop D/Q crossing clock domain boundary | D input changes within (T_setup + T_hold) of receiving clock Synchronizer depth < 2 (single-flop CDC detected) Checked via annotated SDF + CDC analysis | S1 | DMS | severity=S1-Critical, domain=DMS, bug_type=Functional, root_cause_category=CDC_RDC, title=[CDC][Metastability] Single-flop CDC detected on {signal_name} |
| CHK_CDC_NOSYNC_202 Missing Synchronizer | Signals crossing clock domain | Signal crosses clock domain without recognized synchronizer cell Check: src_clk ≠ dst_clk AND no sync cell in path Cell library: dff_sync2, sync_cell_2ff, etc. | S1 | DMS | severity=S1-Critical, domain=DMS, bug_type=Functional, root_cause_category=CDC_RDC, title=[CDC][NoSync] Unsynchronized CDC path {hier_path} |
| CHK_CDC_GRAY_203 Gray Code Violation | Multi-bit counter crossing CDC | Multi-bit bus crossing CDC without Gray encoding OR Gray→Binary decode error (non-adjacent code transition) OR thermometer code CDC without proper handling | S2 | DMS | bug_type=Functional, root_cause_category=CDC_RDC, title=[CDC][GrayCode] Multi-bit CDC without gray encoding on {bus_name} |
| CHK_RST_ASYNC_301 Async Reset Deassert Violation | RST_N, CLK timing | Reset deassert (RST_N rising) occurs within 2 clock periods of CLK edge (violates synchronous deassert requirement) Min measurement window: 1 clock cycle | S2 | Digital | bug_type=Timing, root_cause_category=Methodology_Gap, title=[Reset][AsyncDeassert] Reset deassert not synchronized to clock domain |
| CHK_RST_FANOUT_302 Reset Fanout Excess | RST_N net fanout (synthesized) | Buffered reset net drives > fanout_spec flops without isolation OR reset tree depth > 3 levels without re-synchronization | S3 | Digital | bug_type=Functional, title=[Reset][Fanout] Reset net fanout exceeds methodology limit |
| CHK_RST_PARTIAL_303 Partial Reset Detected | RST domain coverage | Logic block contains flops not in any reset domain OR flop reset state undefined (no reset pin) Affected count > 0 for safety-critical signals | S2 | Digital | bug_type=Functional, root_cause_category=Methodology_Gap, title=[Reset][Partial] Flops without reset detected in {block_name} |
| CHK_FSM_ILLEGAL_401 FSM Illegal State | FSM state register | State register value not in defined state encoding Detected: state = 4'bXXXX where XXXX ∉ {valid_states} Trigger: any simulation step | S1 | Digital | severity=S1-Critical, bug_type=Functional, title=[FSM][IllegalState] FSM reached undefined state {obs_state} in {hier_path} |
| CHK_FSM_UNDEF_TRANS_402 Undefined Transition | FSM next-state logic | Transition from valid state occurs to undefined state OR default case in case statement not handled Triggered by input vector not covered in design | S2 | Digital | bug_type=Functional, title=[FSM][UndefTransition] Undefined state transition from state {from_state} on input {input_vec} |
| CHK_FSM_LOCKOUT_403 Lockout State Reachability | FSM reachability graph (static) | FSM can reach a state with no valid exit transition Checked via reachability analysis on state graph Flagged at elaboration time | S2 | Digital | bug_type=Functional, design_phase=Schematic, title=[FSM][Lockout] Deadlock state reachable in {fsm_name} |
| CHK_TIM_SETUP_501 Setup Violation (SDF annotated) | Reg-to-reg paths with SDF back-annotation | Setup slack < 0ps on any path in post-layout netlist Source: SDF back-annotated GLS Clock: {clk_name} @ {period}ns | S1 | Digital | severity=S1-Critical, bug_type=Timing, design_phase=GLS, title=[Timing][Setup] Negative setup slack {slack}ps on path {startpoint}→{endpoint} |
| CHK_TIM_HOLD_502 Hold Violation (SDF annotated) | Reg-to-reg paths | Hold slack < 0ps (any corner) Critical: hold violations not fixable post-tapeout | S1 | Digital | severity=S1-Critical, bug_type=Timing, root_cause_category=Timing_Closure, title=[Timing][Hold] Negative hold slack {slack}ps – unfixable post-tapeout |
| CHK_TIM_GLITCH_503 Path Sensitization Glitch | Combinational output during transitions | Output glitches > glitch_spec_width detected Measure: output changes > 1 time between stable input and stable output | S3 | Digital | bug_type=Timing, title=[Timing][Glitch] Combinational glitch {width}ps on {signal} during transition |
| CHK_PWR_SPIKE_601 Dynamic Power Spike | VDD current drawn during activity | Instantaneous I_VDD > I_peak_spec Measure: 10ns rolling window average At max frequency, all bits switching | S2 | Digital | bug_type=Power, title=[Power][Spike] Dynamic current spike {i_peak}mA exceeds spec {i_spec}mA |
| CHK_PWR_MULTIVT_602 Multi-Vt Cell Violation | Cell Vt assignment vs timing path criticality | HVT cell on critical path with slack < slack_margin OR LVT cell on non-critical path (leakage waste) Extracted from liberty cell classification | S3 | Digital | bug_type=Power, title=[Power][MultiVt] Incorrect Vt cell assignment on path {path_id} |
| CHK_PWR_ISOLATION_603 Isolation Cell Missing | Power domain crossing signals | Signal crosses from gated to always-on domain without isolation cell Checked against UPF/CPF power intent AND cell type != ISO_* | S2 | Digital | severity=S2-High, bug_type=Power, root_cause_category=Methodology_Gap, title=[Power][Isolation] Missing isolation cell on {signal_name} crossing {from_pd}→{to_pd} |
| CHK_FUNC_CTR_OVF_701 Counter Overflow | Counter output bits | Counter reaches max value and wraps unexpectedly OR carry/overflow flag not asserted at terminal count Check: saturating vs wrapping behavior per spec | S2 | Digital | bug_type=Functional, title=[Functional][Counter] Unexpected counter overflow/wrap in {inst_path} |
| CHK_FUNC_FIFO_702 FIFO Overflow / Underflow | FIFO full/empty flags + data | Write while full (overflow) without write-enable gate OR read while empty (underflow) Check: flag timing vs data valid | S2 | Digital | bug_type=Functional, title=[FIFO][{ov_type}] FIFO {ov_type} in {inst_path} – data integrity at risk |
| CHK_FUNC_ECC_703 Parity / ECC Failure | Syndrome bits, corrected/detected flags | Uncorrectable error detected (double-bit for SEC-DED) OR parity mismatch on protected bus Triggered by injected fault or natural X-prop | S1 | Digital | severity=S1-Critical, bug_type=Functional, title=[ECC][UncorrectableError] ECC UECC event in {mem_name} at address {addr} |
| CHK_SVA_VACUOUS_801 SVA Vacuity Detected | SVA property evaluation count | Assertion antecedent never evaluates TRUE across full regression I.e.: property fires 0 times across all tests Vacuity = silent false-green | S2 | Digital | bug_type=Coverage_Hole, root_cause_category=Methodology_Gap, title=[SVA][Vacuity] Assertion {prop_name} never triggered – coverage gap, description=Assertion exists but antecedent never satisfied |
| CHK_SVA_NEVER_802 Assertion Never Triggered | SVA assertion fire count | Assertion property fires 0 times AND antecedent fires > 0 times (i.e.: the behavior being checked never occurs) Potential missing test or dead code | S3 | Digital | bug_type=Coverage_Hole, title=[SVA][NeverTriggered] Property {prop_name} antecedent fires but consequent never reached |

## 3.4  Mixed-Signal Interface Checker Definitions

| MIXED-SIGNAL INTERFACE CHECKERS — CHK_ADC / DAC / COMP / LVLSHIFT / HANDSHAKE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Checker ID / Name | Monitored Signal(s) | Threshold / Condition | Sev | Domain | Auto-Prefill Fields |
| CHK_ADC_DNL_901 ADC DNL Out-of-Spec | ADC output code histogram | DNL > +0.5 LSB OR DNL < −0.5 LSB (missing code if < −1 LSB) Measure: histogram test N=2^(N+4) samples Code density method | S2 | Interface | domain=Interface, bug_type=Parametric, title=[ADC][DNL] Differential nonlinearity violation at code {code_hex}, observed_behavior=DNL={dnl_lsb}LSB spec=±0.5LSB |
| CHK_ADC_INL_902 ADC INL Out-of-Spec | ADC transfer function | INL > INL_spec_lsb (end-point fit) OR INL > INL_spec_lsb (best-fit line) Extracted from ramp input test | S2 | Interface | bug_type=Parametric, title=[ADC][INL] Integral nonlinearity exceeds spec, observed_behavior=INL_max={inl_max}LSB spec={inl_spec}LSB |
| CHK_ADC_MISSING_903 Missing Code Detected | ADC output code distribution | Any output code has 0 occurrences over N_min samples N_min = 8 × expected_density Implies DNL < −1 LSB at that code | S1 | Interface | severity=S1-Critical, bug_type=Parametric, title=[ADC][MissingCode] Missing output code {code_hex} detected – DNL < −1LSB |
| CHK_ADC_GLITCH_904 Conversion Glitch | ADC output bus during conversion | Output code changes more than once between consecutive valid samples OR output changes during HOLD phase Glitch width > 1 sample period | S2 | Interface | bug_type=Functional, title=[ADC][Glitch] Output glitch detected during conversion at t={t_glitch} |
| CHK_DAC_SETTLE_905 DAC Settling Violation | DAC VOUT after code change | T_settle to 0.5 LSB of final value > settle_spec Measure: code change → VOUT within error band Test: major carry transition (0x7FFF → 0x8000) | S2 | Interface | bug_type=Functional, title=[DAC][Settling] DAC output settling time {t_settle}us exceeds spec {spec_us}us |
| CHK_DAC_GLITCH_906 DAC Glitch Energy Excess | DAC VOUT glitch during code transition | Glitch energy (∫¦Vglitch¦×dt) > glitch_energy_spec Measure: major-carry transition glitch impulse Unit: nV·s or LSB·clocks | S2 | Interface | bug_type=Noise_Margin, title=[DAC][GlitchEnergy] Major-carry glitch energy {energy}nVs exceeds spec {spec}nVs |
| CHK_COMP_META_907 Comparator Metastability Window | COMP output transition time | Output transition time > T_meta_spec Input differential < Vos + hysteresis AND close to switching threshold Risk window: ¦Vin+ − Vin−¦ < 10mV | S1 | Interface | severity=S1-Critical, bug_type=Functional, root_cause_category=Design_Intent_Misunderstanding, title=[Comp][Metastability] Output transition time {t_trans}ns in metastable region |
| CHK_COMP_HYST_908 Comparator Hysteresis Check | COMP rising and falling thresholds | Hysteresis = V_th_rise − V_th_fall Hysteresis < hyst_min_spec (insufficient noise immunity) OR Hysteresis > hyst_max_spec (excessive dead-band) | S2 | Interface | bug_type=Parametric, title=[Comp][Hysteresis] Hysteresis {hyst_mv}mV outside spec [{hmin}, {hmax}]mV |
| CHK_LVLSHIFT_UNDEF_909 Level-Shifter Undefined Logic Level | Level-shifter output voltage | Output voltage in undefined logic band: VOL > V_IL_max (0.3×VDD_dst) AND VOH < V_IH_min (0.7×VDD_dst) Sustained > 2ns | S1 | Interface | severity=S1-Critical, domain=Interface, bug_type=Functional, root_cause_category=Integration_Assumption, title=[LvlShift][Undefined] Level-shifter output in undefined band {vh}V at {hier} |
| CHK_ANA_DIG_HANDSHAKE_910 Analog-Digital Handshake Timing | ready/valid digital vs analog settling time | Digital VALID asserted before analog output settled to within 0.5 LSB Separation: T_valid_assert − T_analog_settle < T_margin_spec Margin_spec typically 2 sample periods | S2 | Interface | bug_type=Timing, root_cause_category=Integration_Assumption, title=[Interface][Handshake] Digital valid asserted before analog settled, t_settle={t_settle} t_valid={t_valid} |

## 3.5  Checker Event Record — Metadata Schema

Every checker, regardless of domain, emits a structured CheckerEvent record on trigger. This is the wire format consumed by the VBIP ingestion API at POST /api/v1/bugs/auto-file.

| Field | Type | Description & Semantics |
| --- | --- | --- |
| checker_id | string | Unique identifier. Format: CHK_{DOMAIN}_{ABBREV}_{SEQ:03d}. Immutable. References checker registry. |
| checker_category | enum | SupplyRail ¦ LDO ¦ PLL ¦ CurrentMonitor ¦ OTA ¦ ESD ¦ Bias ¦ MonteCarlo ¦ Protocol_AHB ¦ Protocol_APB ¦ Protocol_AXI ¦ CDC ¦ Reset ¦ FSM ¦ Timing ¦ Power ¦ Functional ¦ SVA ¦ ADC ¦ DAC ¦ Comparator ¦ LevelShifter ¦ Handshake |
| domain | enum | AMS ¦ Digital ¦ DMS ¦ Interface ¦ System-Level |
| signal_name | string | Primary signal that triggered the checker. Hierarchical: block.sub.sig_name |
| hierarchy_path | string | Full DUT path to failing instance. Separator: '/'. Example: chip/pmu/ldo_core/error_amp |
| time_of_occurrence | float | Simulation time when checker fired. Unit defined in t_unit field. |
| t_unit | enum | fs ¦ ps ¦ ns ¦ us ¦ ms ¦ s |
| observed_value | float | Measured value of the monitored quantity at time_of_occurrence. In natural units of the checker. |
| threshold_value | float | Specification threshold (max or min) that was violated. Same unit as observed_value. |
| deviation_percent | float | ( ¦observed_value − threshold_value¦ / ¦threshold_value¦ ) × 100.0. Always positive. |
| corner | string | Process corner. Example: tt, ff, ss, fnsp, sfnp |
| temperature | float | Simulation temperature in Celsius. |
| pvt_point | string | Composite PVT. Format: {corner}/{voltage}V/{temp}C. Example: tt/1.8V/27C |
| waveform_marker_commands | list | List of TCL commands to reproduce the waveform state in SimVision. Each entry: string TCL command. Executed in sequence to add signals, set cursor, add markers, and zoom to failure window. |
| suggested_bug_severity | enum | S1-Critical ¦ S2-High ¦ S3-Medium ¦ S4-Low. Based on checker-level severity mapping. Overridable by DV engineer at filing. |
| suggested_root_cause_category | enum | One of the root_cause_category enum values from Section 1. Pre-populated from checker knowledge base. |
| auto_prefill_fields | object | Key-value map of VBIP bug template fields pre-populated from checker event. Covers: title, domain, bug_type, primary_signals, t_start, t_peak, t_end, observed_behavior, failure_conditions, checker_name, checker_file, checker_line, assertion_text. |
| run_metadata | object | Sub-fields: tb_name, test_name, seed, corner, pdk_version, simulator_version, run_id. Auto-populated from simulation environment. |
| confidence_score | float | Prefill confidence: 0.0–1.0. High (>0.85) = auto-file eligible. Medium (0.5–0.85) = human review. Low (<0.5) = manual filing recommended. |

## 3.6  CheckerEvent JSON Wire Format — Canonical Example

This example shows a fully-populated CheckerEvent emitted by CHK_LDO_STAB_008 firing in a post-layout simulation corner:

{

"checker_id":        "CHK_LDO_STAB_008",

"checker_category":  "LDO",

"domain":            "AMS",

"signal_name":       "ldo_core.vout",

"hierarchy_path":    "chip_top/pmu_ss/ldo_core",

"time_of_occurrence": 45820.5,

"t_unit":            "ns",

"observed_value":    0.412,

"threshold_value":   0.500,

"deviation_percent": 17.6,

"corner":            "ff",

"temperature":       125.0,

"pvt_point":         "ff/1.98V/125C",

"waveform_marker_commands": [

"add_wave chip_top/pmu_ss/ldo_core/vout",

"add_wave chip_top/pmu_ss/ldo_core/vfb",

"add_wave chip_top/pmu_ss/ldo_core/vref",

"add_wave chip_top/pmu_ss/ldo_core/vcomp",

"set_cursor 45820.5 ns",

"add_marker 45820.5 ns CHECKER_FIRE",

"zoom_window 45700.0 46000.0 ns"

],

"suggested_bug_severity":         "S1-Critical",

"suggested_root_cause_category":  "Design_Intent_Misunderstanding",

"confidence_score": 0.91,

"auto_prefill_fields": {

"identity.title":           "[LDO][Stability] Phase margin proxy < 45 deg - oscillation risk at ff/125C",

"classification.domain":    "AMS",

"classification.bug_type":  "Functional",

"classification.design_phase": "Post-layout",

"classification.severity":  "S1-Critical",

"design_ref.hierarchy_path": "chip_top/pmu_ss/ldo_core",

"design_ref.primary_signals": ["ldo_core.vout", "ldo_core.vfb", "ldo_core.vcomp"],

"design_ref.t_start":       45700.0,

"design_ref.t_peak":        45820.5,

"design_ref.t_end":         46000.0,

"design_ref.t_unit":        "ns",

"design_ref.corner_pvt":    "ff/1.98V/125C",

"description.observed_behavior":

"Damping ratio = 0.41 (proxy from step response). Phase margin < 45 deg estimated. Corner: ff/1.98V/125C.",

"description.checker_name": "CHK_LDO_STAB_008",

"description.assertion_text":

"assert (damping_ratio >= 0.5) else $error(CHECKER:CHK_LDO_STAB_008)",

"project.schedule_impact":  "Blocking-Tapeout"

},

"run_metadata": {

"tb_name":           "tb_ldo_postlayout",

"test_name":         "test_ldo_load_step_worst_corner",

"seed":              48291,

"corner":            "ff",

"pdk_version":       "tsmc16ffc_v1.7",

"simulator_version": "spectre 21.1.0",

"run_id":            "run_20240315_142237_ff_125C"

}

}

## 3.7  Waveform Marker File Format

Each checker generates a waveform_marker_file — a TCL replay script loadable in SimVision via File > Load > Script. The file reconstructs the exact waveform debug context.

# VBIP Waveform Marker File

# Checker: CHK_LDO_STAB_008  |  Bug: CRNX-LDO-A-0042

# Generated: 2024-03-15T14:22:37Z  |  Run: run_20240315_142237_ff_125C

# Load in SimVision: File > Load > Script

# 1. Add signals to waveform window

add_wave -radix real chip_top/pmu_ss/ldo_core/vout

add_wave -radix real chip_top/pmu_ss/ldo_core/vfb

add_wave -radix real chip_top/pmu_ss/ldo_core/vref

add_wave -radix real chip_top/pmu_ss/ldo_core/vcomp

add_wave -radix real chip_top/pmu_ss/ldo_core/iload

# 2. Set simulation cursor to failure time

set_time_cursor 45820.5ns

# 3. Add named markers at key events

add_marker -name "CHECKER_FIRE"     -time 45820.5ns -color red

add_marker -name "STEP_INPUT_START" -time 45700.0ns -color orange

add_marker -name "SETTLE_DEADLINE"  -time 46000.0ns -color yellow

# 4. Zoom to failure window (±200ns around event)

zoom_to_window 45600.0ns 46100.0ns

# 5. Apply measurement annotations

measure_delta -from "STEP_INPUT_START" -to "CHECKER_FIRE" -label "Ringing onset"

# SECTION 4 — LOG PARSER ENGINE SPECIFICATION

This section specifies the production-quality Python log parser module: vbip_log_parser. It processes raw simulator and STA tool logs from Xcelium, ngspice, Spectre, Tempus, and PrimeTime, and produces structured FailureRecord objects and VBIP PrefillPayload instances ready for API submission.

Design Constraint The parser must handle logs exceeding 2GB (full-chip regression). All I/O is streaming — no full-file reads. Pattern matching is compiled-regex only (no interpreted string scanning). Deduplication operates in O(1) memory per unique hash via a rolling LRU cache.

## 4.1  Processing Pipeline

| Stage | Description |
| --- | --- |
| 1. Ingestion | Read raw log file. Detect simulator type from header patterns. Stream line-by-line for large files (>500MB). Split into logical segments: header, simulation body, summary footer. |
| 2. Tokenization | Scan each line for structural tokens: timestamp, severity keyword, hierarchy path, signal name, numeric value. Build token stream with (token_type, value, line_no, raw_text) tuples. |
| 3. Pattern Matching | Apply ordered regex pattern library per detected simulator type. Each pattern match yields a RawMatch(pattern_id, groups, line_no, raw_text). Patterns ordered by specificity (most specific first). |
| 4. Entity Extraction | From RawMatch groups, extract typed entities: SignalRef, HierPath, TimeValue, NumericValue, CornerRef, CheckerRef. Apply normalization (unit conversion, path canonicalization). |
| 5. Record Assembly | Group related RawMatch objects into a single FailureRecord. Multi-line errors (STA path reports) assembled by state machine tracking path start/end markers. |
| 6. Severity Classification | Map pattern_id + extracted entities to severity level using classifier rules. Override with explicit $fatal (→S1) or tool license error (→S4) mappings. |
| 7. Deduplication | Hash-based deduplication: hash(checker_id + signal + hierarchy + time_bucket). Merge duplicates within ±10ns time window. Track occurrence_count. |
| 8. Prefill Generation | Map FailureRecord fields → VBIP bug template fields. Compute confidence_score. Generate PrefillPayload. Attach run metadata from parsed header. |

## 4.2  Python Module Structure

# vbip_log_parser/

# ├── __init__.py

# ├── parser.py          ← Main entry point: LogParser class

# ├── patterns.py        ← Compiled regex pattern library (all simulators)

# ├── tokenizer.py       ← Line-level tokenization and simulator detection

# ├── extractor.py       ← Entity extraction from RawMatch groups

# ├── classifier.py      ← Severity classification rules

# ├── deduplicator.py    ← Hash-based dedup with LRU cache

# ├── prefill.py         ← FailureRecord → PrefillPayload mapping

# ├── models.py          ← Dataclass definitions (FailureRecord, PrefillPayload, etc.)

# └── tests/

#     ├── fixtures/      ← Sample log lines per simulator (one file per pattern)

#     └── test_parser.py ← pytest suite (≥1 test per pattern)

## 4.3  Data Model — Python Dataclasses

All dataclasses use Python 3.10+ syntax with full type annotations. Serialization via dataclasses.asdict() + json.dumps().

# models.py

from __future__ import annotations

from dataclasses import dataclass, field

from typing import Optional

from enum import Enum

class SimulatorType(str, Enum):

XCELIUM   = "xcelium"

NGSPICE   = "ngspice"

SPECTRE   = "spectre"

TEMPUS    = "tempus"

PRIMETIME = "primetime"

UVM       = "uvm"         # UVM messages embedded in Xcelium/VCS log

UNKNOWN   = "unknown"

class FailureSeverity(str, Enum):

S1_CRITICAL = "S1-Critical"

S2_HIGH     = "S2-High"

S3_MEDIUM   = "S3-Medium"

S4_LOW      = "S4-Low"

class FailureCategory(str, Enum):

ASSERTION_FAIL    = "assertion_fail"

FATAL_ERROR       = "fatal_error"

RUNTIME_ERROR     = "runtime_error"

TIMING_VIOLATION  = "timing_violation"

CONVERGENCE_FAIL  = "convergence_fail"

MEASURE_FAIL      = "measure_fail"

LICENSE_ERROR     = "license_error"

TOOL_WARNING      = "tool_warning"

UVM_ERROR         = "uvm_error"

UVM_FATAL         = "uvm_fatal"

X_PROPAGATION     = "x_propagation"

RACE_CONDITION    = "race_condition"

SPICE_OP_FAIL     = "spice_op_fail"

MODEL_NOT_FOUND   = "model_not_found"

SUBCIRCUIT_MISSING= "subcircuit_missing"

@dataclass

class SignalRef:

"""A resolved signal reference with full hierarchy."""

signal_name:    str

hierarchy_path: str

full_path:      str       # hierarchy_path + "/" + signal_name

bus_width:      Optional[int] = None

@dataclass

class TimeValue:

"""A simulation time with explicit unit."""

value: float

unit:  str    # fs | ps | ns | us | ms | s

def to_ns(self) -> float:

factors = {"fs":1e-6, "ps":1e-3, "ns":1.0, "us":1e3, "ms":1e6, "s":1e9}

return self.value * factors.get(self.unit, 1.0)

@dataclass

class TimingPath:

"""Extracted from STA tool (Tempus / PrimeTime) path report."""

startpoint:       str

endpoint:         str

path_group:       str

clock_name:       str

slack_ps:         float         # negative = violation

data_arrival_ps:  float

data_required_ps: float

slack_type:       str           # "setup" | "hold"

corner:           Optional[str] = None

path_lines:       list[str] = field(default_factory=list)  # raw path detail lines

@dataclass

class FailureRecord:

"""

One structured failure event extracted from simulator/STA log.

Maps 1:1 to a candidate VBIP bug report.

"""

# Core identity

record_id:          str           # UUID4 generated at extraction time

simulator_type:     SimulatorType

failure_category:   FailureCategory

severity:           FailureSeverity

# Location

source_file:        str           # Log file path

line_number:        int           # Line number in source_file

raw_text:           str           # Original log line(s) — max 20 lines

# Signal and hierarchy

signal:             Optional[SignalRef] = None

hierarchy_path:     Optional[str]       = None

block_name:         Optional[str]       = None    # Extracted top-level block

# Timing

time_of_occurrence: Optional[TimeValue] = None

# Values

observed_value:     Optional[float]     = None

threshold_value:    Optional[float]     = None

value_unit:         Optional[str]       = None    # V, A, Hz, ps, etc.

# Checker reference

checker_name:       Optional[str]       = None

checker_file:       Optional[str]       = None

checker_line:       Optional[int]       = None

assertion_text:     Optional[str]       = None

# STA-specific

timing_path:        Optional[TimingPath]= None

# Run metadata

testbench_name:     Optional[str]       = None

test_name:          Optional[str]       = None

seed:               Optional[int]       = None

corner:             Optional[str]       = None

temperature:        Optional[float]     = None

pdk_version:        Optional[str]       = None

simulator_version:  Optional[str]       = None

run_id:             Optional[str]       = None

# Stop reason

sim_stop_reason:    Optional[str]       = None   # assertion | $finish | $fatal | timeout | convergence

# UVM

uvm_component:      Optional[str]       = None

uvm_message_id:     Optional[str]       = None

# Dedup

dedup_hash:         Optional[str]       = None   # SHA256(checker+signal+hier+time_bucket)

occurrence_count:   int = 1

@dataclass

class PrefillPayload:

"""

Maps a FailureRecord to VBIP bug template fields.

Submitted to POST /api/v1/bugs/auto-file as the prefill_fields object.

"""

# Mandatory VBIP fields auto-populated

title:               str

domain:              str           # AMS | DMS | Digital | Interface | System-Level

bug_type:            str           # VBIP classification enum

design_phase:        str           # Default: inferred from run context

severity:            str           # S1-Critical .. S4-Low

priority:            str           # P1..P4

# Design reference fields

hierarchy_path:      Optional[str]       = None

primary_signals:     list[str]           = field(default_factory=list)

corner_pvt:          Optional[str]       = None

t_start:             Optional[float]     = None

t_peak:              Optional[float]     = None

t_end:               Optional[float]     = None

t_unit:              str = "ns"

# Description

expected_behavior:   Optional[str]       = None

observed_behavior:   Optional[str]       = None

failure_conditions:  dict                = field(default_factory=dict)

reproduction_steps:  list[str]           = field(default_factory=list)

log_excerpt:         Optional[str]       = None

# Checker linkage

checker_name:        Optional[str]       = None

checker_file:        Optional[str]       = None

checker_line:        Optional[int]       = None

assertion_text:      Optional[str]       = None

# Project fields

schedule_impact:     str = "On-Track"

estimated_effort_hrs: float = 4.0

# Metadata for API

confidence_score:    float = 0.5         # 0.0–1.0

source_record_id:    Optional[str]       = None   # FailureRecord.record_id

suggested_root_cause_category: Optional[str] = None

## 4.4  Pattern Library — Complete Regex Definitions

All patterns are pre-compiled at module import time. Named capture groups map directly to FailureRecord fields. Test fixture lines shown after each pattern.

### 4.4.1  Xcelium Patterns

# patterns.py  —  Xcelium section

import re

XCELIUM_PATTERNS: dict[str, re.Pattern] = {}

# ── Assertion failure ($error / assert / SVA)

# Example: xmsim: *E,ASRTST (tb_ldo.sv,142): (time 45820500 FS) Checker CHK_LDO_STAB_008 failed

XCELIUM_PATTERNS["xcel_assert_fail"] = re.compile(

r"xmsim:\s+\*(?P<severity>[EWF]),ASRTST\s+"

r"\((?P<file>[^,]+),(?P<line>\d+)\):\s+"

r"\(time\s+(?P<time>\d+(?:\.\d+)?)\s+(?P<t_unit>FS|PS|NS|US|MS|S)\)\s+"

r"(?P<message>.+)",

re.IGNORECASE

)

# Fixture:

# xmsim: *E,ASRTST (tb_ldo.sv,142): (time 45820500 FS) Checker CHK_LDO_STAB_008 triggered

# xmsim: *E,ASRTST (assertions/proto_chk.sv,89): (time 1023400 PS) AXI handshake timeout

# ── $error / $fatal system task

# Example: tb_ldo.sv:89: $fatal triggered: LDO VOUT below dropout limit

XCELIUM_PATTERNS["xcel_dollar_fatal"] = re.compile(

r"(?P<file>[\w./]+\.s[v]?):"

r"(?P<line>\d+):\s+"

r"\$(?P<level>fatal|error)\s+(?:triggered:?)?\s*"

r"(?P<message>.+)",

re.IGNORECASE

)

# Fixture:

# tb_pll.sv:203: $fatal triggered: PLL lock not achieved within spec

# tb_axi.sv:511: $error triggered: Write response ID mismatch

# ── X-propagation warning

# Example: xmsim: *W,TPXDEP (chip_top.v,7821): (time 10020 NS) X detected on chip_top.pmu.ldo.enable

XCELIUM_PATTERNS["xcel_x_prop"] = re.compile(

r"xmsim:\s+\*W,TPXDEP\s+"

r"\((?P<file>[^,]+),(?P<line>\d+)\):\s+"

r"\(time\s+(?P<time>\d+(?:\.\d+)?)\s+(?P<t_unit>\w+)\)\s+"

r"X detected on\s+(?P<signal_path>[\w./\[\]]+)",

re.IGNORECASE

)

# Fixture:

# xmsim: *W,TPXDEP (chip_top.v,7821): (time 10020 NS) X detected on chip_top.pmu.ldo.enable

# xmsim: *W,TPXDEP (fsm.v,44): (time 55320 NS) X detected on fsm_inst.state_reg[2:0]

# ── Race condition

# Example: xmsim: *W,RACEC (clk_div.v,334): (time 200 NS) Race condition on net clk_div/q_out

XCELIUM_PATTERNS["xcel_race"] = re.compile(

r"xmsim:\s+\*W,RACEC\s+"

r"\((?P<file>[^,]+),(?P<line>\d+)\):\s+"

r"\(time\s+(?P<time>\d+(?:\.\d+)?)\s+(?P<t_unit>\w+)\)\s+"

r"Race condition on net\s+(?P<signal_path>[\w./\[\]]+)",

re.IGNORECASE

)

# Fixture:

# xmsim: *W,RACEC (clk_div.v,334): (time 200 NS) Race condition on net clk_div/q_out

# ── Timing violation (SDF back-annotated GLS)

# Example: xmsim: *E,TIMVIO (chip_top_gl.v,1204): (time 1002000 PS) Setup time violation on ff_vout/D: slack=-42ps

XCELIUM_PATTERNS["xcel_timing_vio"] = re.compile(

r"xmsim:\s+\*E,TIMVIO\s+"

r"\((?P<file>[^,]+),(?P<line>\d+)\):\s+"

r"\(time\s+(?P<time>\d+(?:\.\d+)?)\s+(?P<t_unit>\w+)\)\s+"

r"(?P<vio_type>Setup|Hold) time violation on\s+(?P<signal_path>[\w./\[\]]+)"

r"(?:.*slack=(?P<slack>-?\d+)ps)?",

re.IGNORECASE

)

# Fixture:

# xmsim: *E,TIMVIO (chip_top_gl.v,1204): (time 1002000 PS) Setup time violation on ff_vout/D: slack=-42ps

# xmsim: *E,TIMVIO (chip_top_gl.v,882): (time 1004500 PS) Hold time violation on sync_ff/D: slack=-8ps

# ── Tool license error (distinguish from design error)

# Example: ERROR: License checkout failed for tool xmvlog/64 from server licserv01

XCELIUM_PATTERNS["xcel_license_error"] = re.compile(

r"ERROR:\s+License checkout failed for tool\s+(?P<tool>[\w/]+)"

r"(?:\s+from server\s+(?P<server>[\w.]+))?",

re.IGNORECASE

)

### 4.4.2  ngspice Patterns

# ── ngspice convergence failure

# Example: Error: doAnalysis: Singular matrix detected at node vout#branch

XCELIUM_PATTERNS["ngspice_singular"] = re.compile(

r"Error:\s+doAnalysis:\s+Singular matrix detected at node\s+(?P<node>[\w#.]+)",

re.IGNORECASE

)

# Fixture: Error: doAnalysis: Singular matrix detected at node ldo_core/vout#branch

# ── ngspice iteration limit

# Example: Warning: Transient: TIMESTEP = 1.0e-15 (minimum), at T = 45.8205e-09

XCELIUM_PATTERNS["ngspice_timestep_min"] = re.compile(

r"Warning:\s+(?P<analysis>Transient|AC|DC):\s+"

r"TIMESTEP\s*=\s*(?P<step>[\d.e+\-]+)\s+\(minimum\)\s*,\s+"

r"at\s+T\s*=\s*(?P<time>[\d.e+\-]+)",

re.IGNORECASE

)

# Fixture: Warning: Transient: TIMESTEP = 1.0e-15 (minimum), at T = 45.8205e-09

# ── ngspice node voltage divergence

# Example: Warning: VOUT has diverged: value = 1.2345e+04 V

XCELIUM_PATTERNS["ngspice_diverge"] = re.compile(

r"Warning:\s+(?P<node>[\w./]+)\s+has\s+diverged:\s+"

r"value\s*=\s*(?P<value>[\d.e+\-]+)\s*(?P<unit>V|A|Hz)?",

re.IGNORECASE

)

# Fixture: Warning: VOUT has diverged: value = 1.2345e+04 V

# ── ngspice .measure FAIL

# Example: ldo_load_undershoot  =  FAILED

XCELIUM_PATTERNS["ngspice_measure_fail"] = re.compile(

r"^\s*(?P<measure_name>[\w_]+)\s*=\s*(?:FAILED|failed|FAIL|fail)\s*$",

re.MULTILINE

)

# Fixture: ldo_load_undershoot  =  FAILED

# Fixture: pll_lock_time        =  FAILED

# ── SPICE operating point failure (Spectre / ngspice)

# Example: ERROR (SPECTRE-16028): Can't find DC operating point.

XCELIUM_PATTERNS["spice_op_fail"] = re.compile(

r"ERROR\s*(?:\([\w-]+\))?:\s*Can'?t find\s+DC operating point",

re.IGNORECASE

)

# ── Model not found

# Example: Warning: model 'nmos_lvt_v2' not found in PDK library

XCELIUM_PATTERNS["spice_model_missing"] = re.compile(

r"(?:Warning|Error):\s+model\s+'?(?P<model_name>[\w_]+)'?\s+not found",

re.IGNORECASE

)

# Fixture: Warning: model 'nmos_hvt_v3' not found in PDK library

# ── Subcircuit not found

# Example: Error: Subcircuit 'LDO_CORE_v2' used by X1 but not defined

XCELIUM_PATTERNS["spice_subckt_missing"] = re.compile(

r"Error:\s+Subcircuit\s+'?(?P<subckt>[\w_]+)'?\s+used by\s+(?P<inst>[\w]+)\s+but not defined",

re.IGNORECASE

)

### 4.4.3  Tempus / PrimeTime STA Patterns

# ── Tempus/PT Setup violation (start of path report block)

# Example:

#   Slack (VIOLATED) :           -0.042ns  (data required time - data arrival time)

#   Source:                      ldo_ctrl/ff_vout/CLK  (rising edge-triggered flip-flop clocked by CLK_CORE)

#   Destination:                 ldo_ctrl/ff_iload/D

#   Path Group:                  CLK_CORE

#   Path Type:                   max

XCELIUM_PATTERNS["sta_slack_violated"] = re.compile(

r"Slack\s+\(VIOLATED\)\s*:\s*(?P<slack>-[\d.]+)\s*(?P<unit>ns|ps)",

re.IGNORECASE

)

# Fixture: Slack (VIOLATED) :           -0.042ns  (data required time - data arrival time)

XCELIUM_PATTERNS["sta_startpoint"] = re.compile(

r"Source\s*:\s*(?P<startpoint>[\w./\[\]]+)",

re.IGNORECASE

)

XCELIUM_PATTERNS["sta_endpoint"] = re.compile(

r"Destination\s*:\s*(?P<endpoint>[\w./\[\]]+)",

re.IGNORECASE

)

XCELIUM_PATTERNS["sta_path_type"] = re.compile(

r"Path\s+Type\s*:\s*(?P<path_type>max|min)",

re.IGNORECASE

)

# path_type=max → setup violation; path_type=min → hold violation

XCELIUM_PATTERNS["sta_clock_name"] = re.compile(

r"clocked by\s+(?P<clock_name>[\w_]+)",

re.IGNORECASE

)

# ── Tempus max_transition violation

# Example: Warning: max_transition violation on net ldo_ctrl/vctrl: transition=0.842ns, limit=0.500ns

XCELIUM_PATTERNS["sta_max_transition"] = re.compile(

r"(?:Warning|Error):\s+max_transition violation on net\s+(?P<net>[\w./]+):\s+"

r"transition=(?P<actual>[\d.]+)(?P<unit>ns|ps),\s*limit=(?P<limit>[\d.]+)",

re.IGNORECASE

)

# Fixture: Warning: max_transition violation on net ldo_ctrl/vctrl: transition=0.842ns, limit=0.500ns

# ── Tempus fanout violation

# Example: Warning: max_fanout violation on net clk_buf_3/Y: fanout=248, limit=64

XCELIUM_PATTERNS["sta_max_fanout"] = re.compile(

r"(?:Warning|Error):\s+max_fanout violation on net\s+(?P<net>[\w./]+):\s+"

r"fanout=(?P<actual>\d+),\s*limit=(?P<limit>\d+)",

re.IGNORECASE

)

### 4.4.4  UVM Message Patterns

# ── UVM_ERROR

# Example: UVM_ERROR @ 45820.500 ns: uvm_test_top.env.ldo_agent.monitor [LDO_MON] Vout below spec: 1.742V

XCELIUM_PATTERNS["uvm_error"] = re.compile(

r"UVM_ERROR\s+@\s+(?P<time>[\d.]+)\s+(?P<t_unit>\w+):\s+"

r"(?P<component>[\w.]+)\s+\[(?P<msg_id>[\w_]+)\]\s+(?P<message>.+)",

re.IGNORECASE

)

# Fixture: UVM_ERROR @ 45820.500 ns: uvm_test_top.env.ldo_agent.monitor [LDO_MON] Vout below spec: 1.742V

# Fixture: UVM_ERROR @ 1023400.000 ps: uvm_test_top.env.axi_sb [AXI_SCBD] Response ID mismatch: exp=3 got=5

# ── UVM_FATAL

# Example: UVM_FATAL @ 55000.000 ns: uvm_test_top.env.fsm_chk [FSM_CHK] Illegal state 4'b1011 detected

XCELIUM_PATTERNS["uvm_fatal"] = re.compile(

r"UVM_FATAL\s+@\s+(?P<time>[\d.]+)\s+(?P<t_unit>\w+):\s+"

r"(?P<component>[\w.]+)\s+\[(?P<msg_id>[\w_]+)\]\s+(?P<message>.+)",

re.IGNORECASE

)

# ── UVM summary line (end of test)

# Example: UVM_ERROR :    3  (in simulation)

XCELIUM_PATTERNS["uvm_summary"] = re.compile(

r"UVM_(?P<level>ERROR|FATAL|WARNING)\s*:\s*(?P<count>\d+)\s+\(in simulation\)",

re.IGNORECASE

)

## 4.5  Core Parser Class

# parser.py

from __future__ import annotations

import hashlib, uuid, re

from pathlib import Path

from typing import Iterator

from .models import (FailureRecord, PrefillPayload, SimulatorType,

FailureSeverity, FailureCategory, SignalRef, TimeValue, TimingPath)

from .patterns import XCELIUM_PATTERNS

from .classifier import classify_severity, classify_category

from .prefill import build_prefill_payload

from .deduplicator import Deduplicator

class LogParser:

"""

Streaming log parser for VLSI simulator and STA tool output.

Produces FailureRecord + PrefillPayload pairs for VBIP API submission.

Usage:

parser = LogParser(log_path="/proj/runs/run_20240315/xmsim.log")

for record, prefill in parser.parse():

api_client.submit(prefill)

"""

def __init__(self,

log_path: str | Path,

run_metadata: dict | None = None,

dedup_window_ns: float = 10.0,

auto_file_threshold: float = 0.85):

self.log_path          = Path(log_path)

self.run_metadata      = run_metadata or {}

self.dedup_window_ns   = dedup_window_ns

self.auto_file_threshold = auto_file_threshold

self._dedup            = Deduplicator(window_ns=dedup_window_ns)

self._sim_type         = SimulatorType.UNKNOWN

self._sta_path_buffer: list[str] = []

self._in_sta_path      = False

# ── Simulator detection ──────────────────────────────────────────────────

def _detect_simulator(self, header_lines: list[str]) -> SimulatorType:

header = " ".join(header_lines)

if "xmsim" in header or "xcelium" in header.lower():

return SimulatorType.XCELIUM

if "ngspice" in header.lower():

return SimulatorType.NGSPICE

if "Cadence Spectre" in header:

return SimulatorType.SPECTRE

if "Tempus" in header:

return SimulatorType.TEMPUS

if "PrimeTime" in header:

return SimulatorType.PRIMETIME

return SimulatorType.UNKNOWN

# ── Streaming parse ──────────────────────────────────────────────────────

def parse(self) -> Iterator[tuple[FailureRecord, PrefillPayload]]:

"""

Stream-parse the log file. Yield (FailureRecord, PrefillPayload) for

every unique failure event. Duplicates suppressed by Deduplicator.

"""

header_lines: list[str] = []

header_done = False

with open(self.log_path, "r", encoding="utf-8", errors="replace") as fh:

for line_no, raw_line in enumerate(fh, start=1):

line = raw_line.rstrip()

# Collect header (first 50 lines for simulator detection)

if not header_done:

header_lines.append(line)

if line_no == 50:

self._sim_type = self._detect_simulator(header_lines)

self._ingest_header_metadata(header_lines)

header_done = True

continue

# STA multi-line path accumulation

if self._is_sta_path_start(line):

self._in_sta_path = True

self._sta_path_buffer = [line]

continue

if self._in_sta_path:

self._sta_path_buffer.append(line)

if self._is_sta_path_end(line):

record = self._parse_sta_path_block(self._sta_path_buffer, line_no)

self._in_sta_path = False

self._sta_path_buffer = []

if record:

record = self._dedup.process(record)

if record:

yield record, build_prefill_payload(record, self.run_metadata)

continue

# Single-line pattern matching

record = self._match_line(line, line_no)

if record:

record = self._dedup.process(record)

if record:

yield record, build_prefill_payload(record, self.run_metadata)

# ── Single-line matcher ──────────────────────────────────────────────────

def _match_line(self, line: str, line_no: int) -> FailureRecord | None:

"""Try all patterns in priority order. Return first match as FailureRecord."""

# Priority order: license errors first (avoid misclassifying as design errors)

if m := XCELIUM_PATTERNS["xcel_license_error"].search(line):

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.LICENSE_ERROR,

severity=FailureSeverity.S4_LOW,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

checker_name="TOOL_LICENSE",

)

# UVM FATAL (before UVM ERROR — more specific)

if m := XCELIUM_PATTERNS["uvm_fatal"].search(line):

t = self._parse_time(m.group("time"), m.group("t_unit"))

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=SimulatorType.UVM,

failure_category=FailureCategory.UVM_FATAL,

severity=FailureSeverity.S1_CRITICAL,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

time_of_occurrence=t,

uvm_component=m.group("component"),

uvm_message_id=m.group("msg_id"),

hierarchy_path=self._extract_hier(m.group("component")),

block_name=self._extract_block(m.group("component")),

)

if m := XCELIUM_PATTERNS["uvm_error"].search(line):

t = self._parse_time(m.group("time"), m.group("t_unit"))

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=SimulatorType.UVM,

failure_category=FailureCategory.UVM_ERROR,

severity=FailureSeverity.S2_HIGH,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

time_of_occurrence=t,

uvm_component=m.group("component"),

uvm_message_id=m.group("msg_id"),

hierarchy_path=self._extract_hier(m.group("component")),

block_name=self._extract_block(m.group("component")),

)

if m := XCELIUM_PATTERNS["xcel_assert_fail"].search(line):

t = self._parse_time(m.group("time"), m.group("t_unit"))

sev_char = m.group("severity").upper()

sev = FailureSeverity.S1_CRITICAL if sev_char == "F" else FailureSeverity.S2_HIGH

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.ASSERTION_FAIL, severity=sev,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

time_of_occurrence=t,

checker_file=m.group("file"),

checker_line=int(m.group("line")),

assertion_text=m.group("message"),

checker_name=self._extract_checker_name(m.group("message")),

)

if m := XCELIUM_PATTERNS["xcel_dollar_fatal"].search(line):

sev = (FailureSeverity.S1_CRITICAL

if m.group("level").lower() == "fatal"

else FailureSeverity.S2_HIGH)

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.FATAL_ERROR, severity=sev,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

checker_file=m.group("file"),

checker_line=int(m.group("line")),

assertion_text=m.group("message"),

)

if m := XCELIUM_PATTERNS["xcel_timing_vio"].search(line):

t = self._parse_time(m.group("time"), m.group("t_unit"))

slack = int(m.group("slack")) if m.group("slack") else None

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.TIMING_VIOLATION,

severity=FailureSeverity.S1_CRITICAL,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

time_of_occurrence=t,

signal=self._resolve_signal(m.group("signal_path")),

observed_value=float(slack) if slack else None,

value_unit="ps",

)

if m := XCELIUM_PATTERNS["xcel_x_prop"].search(line):

t = self._parse_time(m.group("time"), m.group("t_unit"))

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.X_PROPAGATION,

severity=FailureSeverity.S2_HIGH,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

time_of_occurrence=t,

signal=self._resolve_signal(m.group("signal_path")),

)

if m := XCELIUM_PATTERNS["ngspice_singular"].search(line):

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=SimulatorType.NGSPICE,

failure_category=FailureCategory.CONVERGENCE_FAIL,

severity=FailureSeverity.S2_HIGH,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

signal=SignalRef(signal_name=m.group("node"),

hierarchy_path="", full_path=m.group("node")),

)

if m := XCELIUM_PATTERNS["ngspice_measure_fail"].search(line):

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=SimulatorType.NGSPICE,

failure_category=FailureCategory.MEASURE_FAIL,

severity=FailureSeverity.S2_HIGH,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

checker_name=m.group("measure_name"),

)

if m := XCELIUM_PATTERNS["spice_model_missing"].search(line):

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.MODEL_NOT_FOUND,

severity=FailureSeverity.S3_MEDIUM,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

assertion_text=f"Model not found: {m.group('model_name')}",

)

if m := XCELIUM_PATTERNS["spice_subckt_missing"].search(line):

return FailureRecord(

record_id=str(uuid.uuid4()), simulator_type=self._sim_type,

failure_category=FailureCategory.SUBCIRCUIT_MISSING,

severity=FailureSeverity.S2_HIGH,

source_file=str(self.log_path), line_number=line_no, raw_text=line,

assertion_text=f"Subcircuit {m.group('subckt')} not defined (used by {m.group('inst')})",

)

return None   # No pattern matched — line is not a failure event

# ── STA path block parser ────────────────────────────────────────────────

def _is_sta_path_start(self, line: str) -> bool:

return bool(XCELIUM_PATTERNS["sta_slack_violated"].search(line))

def _is_sta_path_end(self, line: str) -> bool:

return line.strip().startswith("---") and len(self._sta_path_buffer) > 5

def _parse_sta_path_block(self, lines: list[str],

last_line_no: int) -> FailureRecord | None:

block = " ".join(lines)

m_slack  = XCELIUM_PATTERNS["sta_slack_violated"].search(block)

m_start  = XCELIUM_PATTERNS["sta_startpoint"].search(block)

m_end    = XCELIUM_PATTERNS["sta_endpoint"].search(block)

m_type   = XCELIUM_PATTERNS["sta_path_type"].search(block)

m_clk    = XCELIUM_PATTERNS["sta_clock_name"].search(block)

if not (m_slack and m_start and m_end):

return None

slack_raw = float(m_slack.group("slack"))

unit      = m_slack.group("unit")

slack_ps  = slack_raw * 1000 if unit == "ns" else slack_raw

path_type = m_type.group("path_type") if m_type else "max"

return FailureRecord(

record_id=str(uuid.uuid4()),

simulator_type=self._sim_type,

failure_category=FailureCategory.TIMING_VIOLATION,

severity=FailureSeverity.S1_CRITICAL,

source_file=str(self.log_path),

line_number=last_line_no,

raw_text="\n".join(lines[:20]),

observed_value=slack_ps, value_unit="ps",

timing_path=TimingPath(

startpoint=m_start.group("startpoint"),

endpoint=m_end.group("endpoint"),

path_group="",

clock_name=m_clk.group("clock_name") if m_clk else "unknown",

slack_ps=slack_ps,

data_arrival_ps=0.0, data_required_ps=0.0,

slack_type="setup" if path_type == "max" else "hold",

path_lines=lines,

),

)

# ── Utility methods ──────────────────────────────────────────────────────

@staticmethod

def _parse_time(value: str, unit: str) -> TimeValue:

unit_map = {"FS":"fs","PS":"ps","NS":"ns","US":"us","MS":"ms","S":"s"}

return TimeValue(value=float(value), unit=unit_map.get(unit.upper(), "ns"))

@staticmethod

def _resolve_signal(path: str) -> SignalRef:

parts = path.rsplit("/", 1) if "/" in path else path.rsplit(".", 1)

if len(parts) == 2:

return SignalRef(signal_name=parts[1], hierarchy_path=parts[0], full_path=path)

return SignalRef(signal_name=path, hierarchy_path="", full_path=path)

@staticmethod

def _extract_hier(component: str) -> str:

return component.rsplit(".", 1)[0] if "." in component else component

@staticmethod

def _extract_block(component: str) -> str:

parts = component.split(".")

return parts[-2] if len(parts) >= 2 else parts[0]

@staticmethod

def _extract_checker_name(message: str) -> str | None:

m = re.search(r"CHK_[A-Z_]+_\d+", message)

return m.group(0) if m else None

def _ingest_header_metadata(self, lines: list[str]) -> None:

"""Extract run metadata from log header lines."""

for line in lines:

if "Testbench:" in line:

self.run_metadata.setdefault("tb_name",

line.split("Testbench:")[-1].strip())

if "Seed:" in line or "seed:" in line:

m = re.search(r"[Ss]eed:\s*(\d+)", line)

if m: self.run_metadata.setdefault("seed", int(m.group(1)))

if "Corner:" in line or "corner:" in line:

m = re.search(r"[Cc]orner:\s*(\w+)", line)

if m: self.run_metadata.setdefault("corner", m.group(1))

## 4.6  Prefill Mapping Engine

Translates a FailureRecord to a VBIP PrefillPayload. The mapping rules encode domain knowledge about how simulator failure categories map to VBIP taxonomy.

# prefill.py

from __future__ import annotations

from .models import FailureRecord, PrefillPayload, FailureCategory, SimulatorType

# ── Category → (domain, bug_type, suggested_root_cause) mapping ─────────────

_CATEGORY_MAP: dict[FailureCategory, tuple[str, str, str]] = {

FailureCategory.ASSERTION_FAIL:    ("Digital", "Functional",  "Methodology_Gap"),

FailureCategory.FATAL_ERROR:       ("Digital", "Functional",  "Design_Intent_Misunderstanding"),

FailureCategory.TIMING_VIOLATION:  ("Digital", "Timing",      "Timing_Closure"),

FailureCategory.CONVERGENCE_FAIL:  ("AMS",     "Convergence", "Spec_Ambiguity"),

FailureCategory.MEASURE_FAIL:      ("AMS",     "Parametric",  "Margin_Violation"),

FailureCategory.X_PROPAGATION:     ("Digital", "Functional",  "CDC_RDC"),

FailureCategory.RACE_CONDITION:    ("Digital", "Timing",      "CDC_RDC"),

FailureCategory.UVM_ERROR:         ("DMS",     "Functional",  "Integration_Assumption"),

FailureCategory.UVM_FATAL:         ("DMS",     "Functional",  "Design_Intent_Misunderstanding"),

FailureCategory.SPICE_OP_FAIL:     ("AMS",     "Convergence", "Spec_Ambiguity"),

FailureCategory.MODEL_NOT_FOUND:   ("AMS",     "Tool_PDK",    "Tool_Bug"),

FailureCategory.SUBCIRCUIT_MISSING:("AMS",     "Tool_PDK",    "Methodology_Gap"),

FailureCategory.LICENSE_ERROR:     ("Digital", "Tool_PDK",    "Tool_Bug"),

FailureCategory.TOOL_WARNING:      ("Digital", "Methodology", "Methodology_Gap"),

}

# ── Simulator → design_phase mapping ────────────────────────────────────────

_SIM_PHASE_MAP: dict[SimulatorType, str] = {

SimulatorType.XCELIUM:    "Schematic",    # overridden to GLS if SDF in run_metadata

SimulatorType.NGSPICE:    "Schematic",

SimulatorType.SPECTRE:    "Post-layout",  # Spectre runs typically post-layout

SimulatorType.TEMPUS:     "GLS",

SimulatorType.PRIMETIME:  "GLS",

}

def build_prefill_payload(record: FailureRecord,

run_metadata: dict) -> PrefillPayload:

"""

Map a FailureRecord + run_metadata to a VBIP PrefillPayload.

Returns a payload ready for POST /api/v1/bugs/auto-file.

"""

domain, bug_type, root_cause = _CATEGORY_MAP.get(

record.failure_category,

("Digital", "Functional", "Methodology_Gap")

)

# Override domain for AMS simulator types

if record.simulator_type in (SimulatorType.NGSPICE, SimulatorType.SPECTRE):

domain = "AMS"

# Determine design phase

design_phase = _SIM_PHASE_MAP.get(record.simulator_type, "Schematic")

if run_metadata.get("sdf_annotated"):

design_phase = "GLS"

# Build title from available entities

title = _build_title(record, domain)

# Build observed_behavior string

observed_behavior = _build_observed_behavior(record)

# Build failure_conditions dict

failure_conditions: dict = {}

if record.observed_value is not None and record.value_unit in ("V", "mV"):

failure_conditions["voltage"] = f"{record.observed_value}{record.value_unit}"

elif record.observed_value is not None and record.value_unit in ("A", "mA", "uA"):

failure_conditions["current"] = f"{record.observed_value}{record.value_unit}"

if run_metadata.get("temperature"):

failure_conditions["temperature"] = f"{run_metadata['temperature']}C"

if record.corner:

failure_conditions["pvt_corner"] = record.corner

# Build reproduction steps

repro_steps = _build_repro_steps(record, run_metadata)

# Compute confidence score

confidence = _compute_confidence(record)

# Schedule impact escalation for S1

sched = ("Blocking-Tapeout"

if record.severity.value == "S1-Critical"

else "On-Track")

# Effort estimate heuristic

effort_map = {"S1-Critical": 8.0, "S2-High": 4.0,

"S3-Medium": 2.0, "S4-Low": 1.0}

effort = effort_map.get(record.severity.value, 4.0)

return PrefillPayload(

title=title,

domain=domain,

bug_type=bug_type,

design_phase=design_phase,

severity=record.severity.value,

priority="P1-Immediate" if record.severity.value == "S1-Critical" else "P2-This-Sprint",

hierarchy_path=record.hierarchy_path,

primary_signals=([record.signal.full_path]

if record.signal else []),

corner_pvt=_build_pvt(record, run_metadata),

t_start=(record.time_of_occurrence.to_ns() - 200.0

if record.time_of_occurrence else None),

t_peak=(record.time_of_occurrence.to_ns()

if record.time_of_occurrence else None),

t_end=(record.time_of_occurrence.to_ns() + 200.0

if record.time_of_occurrence else None),

t_unit="ns",

observed_behavior=observed_behavior,

failure_conditions=failure_conditions,

reproduction_steps=repro_steps,

log_excerpt=record.raw_text[:2000],   # max 2000 chars

checker_name=record.checker_name,

checker_file=record.checker_file,

checker_line=record.checker_line,

assertion_text=record.assertion_text,

schedule_impact=sched,

estimated_effort_hrs=effort,

confidence_score=confidence,

source_record_id=record.record_id,

suggested_root_cause_category=root_cause,

)

def _build_title(record: FailureRecord, domain: str) -> str:

block = f"[{record.block_name}]" if record.block_name else "[UNKNOWN_BLOCK]"

feat  = f"[{record.uvm_message_id}]" if record.uvm_message_id else             f"[{record.failure_category.value.replace('_', ' ').title()}]"

if record.timing_path:

behavior = (f"Negative {record.timing_path.slack_type} slack "

f"{record.timing_path.slack_ps:.0f}ps on "

f"{record.timing_path.startpoint.split('/')[-1]}"

f"→{record.timing_path.endpoint.split('/')[-1]}")

elif record.checker_name:

behavior = f"Checker {record.checker_name} triggered"

elif record.signal:

behavior = f"Failure on {record.signal.signal_name}"

else:

behavior = "Simulation failure detected"

return f"{block}{feat} {behavior}"

def _build_observed_behavior(record: FailureRecord) -> str:

parts = []

if record.timing_path:

tp = record.timing_path

parts.append(

f"{tp.slack_type.capitalize()} slack = {tp.slack_ps:.1f}ps "

f"(VIOLATED). Path: {tp.startpoint} → {tp.endpoint}. "

f"Clock: {tp.clock_name}."

)

if record.observed_value is not None:

parts.append(

f"Observed: {record.observed_value}{record.value_unit or ''}. "

f"Threshold: {record.threshold_value}{record.value_unit or ''}."

)

if record.assertion_text:

parts.append(f"Assertion: {record.assertion_text[:200]}")

if record.time_of_occurrence:

parts.append(

f"Time: {record.time_of_occurrence.value}{record.time_of_occurrence.unit}"

)

return " | ".join(parts) if parts else "See log excerpt for details."

def _build_repro_steps(record: FailureRecord, run_metadata: dict) -> list[str]:

steps = []

corner = run_metadata.get("corner", "tt")

tb     = run_metadata.get("tb_name", "<tb_name>")

seed   = run_metadata.get("seed", 0)

steps.append(f"cd $PROJECT_ROOT/sim/{tb}")

steps.append(f"# Reproduce corner: {corner}")

if record.simulator_type == SimulatorType.XCELIUM:

steps.append(

f"xrun -f {tb}.f -sdf_cmd_file sdf_anno.cmd "

f"-seed {seed} +corner={corner} -l repro.log"

)

elif record.simulator_type in (SimulatorType.NGSPICE, SimulatorType.SPECTRE):

steps.append(

f"ngspice -b {tb}.spi -o repro.log  "

f"# Corner: {corner}"

)

steps.append(

f"simvision -waves repro.shm &  "

f"# Then: File > Load Script > {tb}_marker.tcl"

)

if record.checker_file:

steps.append(

f"# Checker definition: {record.checker_file}"

+ (f" line {record.checker_line}" if record.checker_line else "")

)

return steps

def _build_pvt(record: FailureRecord, run_metadata: dict) -> str | None:

corner = record.corner or run_metadata.get("corner")

voltage = run_metadata.get("voltage")

temp = record.temperature or run_metadata.get("temperature")

if corner and voltage and temp is not None:

return f"{corner}/{voltage}V/{temp}C"

return corner

def _compute_confidence(record: FailureRecord) -> float:

"""Score: 0.0–1.0. High = ready for auto-file without human review."""

score = 0.3   # baseline

if record.checker_name:             score += 0.25

if record.hierarchy_path:           score += 0.15

if record.time_of_occurrence:       score += 0.10

if record.signal:                   score += 0.10

if record.timing_path:              score += 0.20

if record.assertion_text:           score += 0.10

if record.failure_category not in (

FailureCategory.TOOL_WARNING,

FailureCategory.LICENSE_ERROR):  score += 0.05

return min(round(score, 2), 1.0)

## 4.7  Deduplicator

# deduplicator.py

from __future__ import annotations

import hashlib

from collections import OrderedDict

from .models import FailureRecord

class Deduplicator:

"""

O(1) memory deduplication using SHA-256 hash of

(checker_id + signal_full_path + hierarchy + time_bucket).

Time bucket = floor(time_ns / window_ns) × window_ns.

"""

def __init__(self, window_ns: float = 10.0, cache_size: int = 10_000):

self._window_ns   = window_ns

self._cache: OrderedDict[str, int] = OrderedDict()   # hash → occurrence_count

self._cache_size  = cache_size

def _compute_hash(self, record: FailureRecord) -> str:

checker  = record.checker_name or record.failure_category.value

signal   = record.signal.full_path if record.signal else ""

hier     = record.hierarchy_path or ""

t_ns     = record.time_of_occurrence.to_ns() if record.time_of_occurrence else 0.0

bucket   = (t_ns // self._window_ns) * self._window_ns

raw      = f"{checker}|{signal}|{hier}|{bucket:.1f}"

return hashlib.sha256(raw.encode()).hexdigest()[:16]

def process(self, record: FailureRecord) -> FailureRecord | None:

"""

Returns the record if it is a new unique event.

Returns None if it is a duplicate (increments occurrence_count in cache).

"""

h = self._compute_hash(record)

record.dedup_hash = h

if h in self._cache:

self._cache[h] += 1

return None    # Duplicate — suppress

# New event — add to cache (evict oldest if full)

if len(self._cache) >= self._cache_size:

self._cache.popitem(last=False)

self._cache[h] = 1

return record

## 4.8  FailureRecord → PrefillPayload Field Mapping Table

Explicit field-by-field mapping for API validation and audit tracing.

| FailureRecord Field | PrefillPayload Field | Mapping Logic |
| --- | --- | --- |
| failure_category + simulator_type | domain + bug_type | _CATEGORY_MAP lookup; AMS override for ngspice/spectre |
| severity | severity + priority | Direct enum value map; S1→P1-Immediate |
| simulator_type + run_metadata.sdf_annotated | design_phase | _SIM_PHASE_MAP; GLS override if SDF present |
| block_name + uvm_message_id + failure_category + signal + timing_path | title | _build_title() — [Block][Feature] Behavior convention |
| hierarchy_path | hierarchy_path | Direct copy |
| signal.full_path | primary_signals[0] | Wrapped in list |
| corner + run_metadata.voltage + temperature | corner_pvt | _build_pvt() — format: corner/voltageV/tempC |
| time_of_occurrence.to_ns() ±200ns | t_start, t_peak, t_end | ±200ns window; t_unit fixed to ns |
| timing_path + observed_value + assertion_text + time_of_occurrence | observed_behavior | _build_observed_behavior() — pipe-separated structured string |
| observed_value + value_unit + run_metadata.temperature + corner | failure_conditions{} | Dict with voltage/current/temperature/pvt_corner keys |
| run_metadata.tb_name + seed + corner + checker_file | reproduction_steps[] | _build_repro_steps() — runnable CLI commands |
| raw_text[:2000] | log_excerpt | Truncated to 2000 chars |
| checker_name, checker_file, checker_line, assertion_text | checker_name/file/line/assertion_text | Direct copy — all optional |
| severity == S1-Critical | schedule_impact | S1 → Blocking-Tapeout; else → On-Track |
| severity (heuristic table) | estimated_effort_hrs | S1=8h, S2=4h, S3=2h, S4=1h baseline estimate |
| _compute_confidence() scoring | confidence_score | 0.3 base + increments for checker_name, hier, time, signal, timing_path |

## 4.9  CLI Interface

The parser ships with a CLI entry point for standalone use and CI/CD pipeline integration:

# Install and run

uv add vbip-log-parser

# Parse a single log — print JSON to stdout

vbip-parse --log xmsim.log --format json

# Parse and auto-submit to VBIP API (confidence ≥ 0.85)

vbip-parse --log xmsim.log \

--api-url https://vbip.internal/api/v1 \

--token $VBIP_TOKEN \

--auto-file-threshold 0.85

# Parse STA log with run metadata override

vbip-parse --log tempus_timing.log \

--sim-type tempus \

--meta corner=ss voltage=1.62V temperature=125 sdf_annotated=true \

--format yaml > failures.yaml

# Batch parse entire regression run directory

vbip-parse --log-dir /proj/runs/regr_20240315/ \

--glob "*.log" \

--dedup-window 10 \

--output-dir /proj/vbip_reports/